A 1000 byte packet is sent from your home to a server. It is first sent over a 10 Mbps wireless link. Once it has been received completely, it is then sent over a 100 Mbps ISP link. The propagation delay of the wireless link is 500 nanoseconds and the propagation delay of the ISP link is 10ms. How long does it take (in milliseconds) for the packet to reach the server?

Packet size = 1000 bytes = 8000 bits

Wireless link rate = 10 Mbps

ISP link rate = 100 Mbps

Propagation delay (wireless) = 500 ns = 0.0005 ms

Propagation delay (ISP) = 10 ms

Step 1: Transmission Delay

1. Wireless Link

Transmission delay=800010×106=0.0008 s=0.8 ms

2. ISP Link

Transmission delay=8000100×106=0.00008 s=0.08 ms

Step 2: Total Delay

Since the second link starts only after the full packet is received (store-and-forward), we add all delays:

Total delay=0.8⏟Wireless Tx+0.0005⏟Wireless Prop+0.08⏟ISP Tx+10⏟ISP Prop
=10.8805 ms

-----------------------------------------------------------

On a wireless ALOHA network stations are maximum 300 Km apart Assume that signals propagate at 3 x 108 m/s Maximum propagation time= Tp = ? Backoff time = TB = ? If K = 1, 2,3 What will be the maximum propagation time and backoff time if stations are 900 Km apart

1. Maximum Propagation Time (Tp)

Formula:

Tp=DistancePropagation speed

Given:

- Distance = 300 km = 300 × 10³ m

- Speed = 3 × 10⁸ m/s

Tp=300×1033×108=10-3 s=1 ms
2. Backoff Time (TB)

In ALOHA:

TB=K×Tp

For:

- K = 1 → TB = 1 × 1 = 1 ms

- K = 2 → TB = 2 × 1 = 2 ms

- K = 3 → TB = 3 × 1 = 3 ms

✅ Backoff times:

- K = 1 → 1 ms

- K = 2 → 2 ms

- K = 3 → 3 ms

b)

When Distance = 900 km

New propagation time:

Tp=900×1033×108=3×10-3 s=3 ms

✅ Maximum propagation time (900 km) = 3 ms

New Backoff Time

TB=K×Tp

- K = 1 → TB = 3 ms

- K = 2 → TB = 6 ms

- K = 3 → TB = 9 ms

| Distance | Tp (ms) | TB (K=1) | TB (K=2) | TB (K=3) |
| --- | --- | --- | --- | --- |
| 300 km | 1 ms | 1 ms | 2 ms | 3 ms |
| 900 km | 3 ms | 3 ms | 6 ms | 9 ms |